CEE  LP-280(3 in 1)Moving Head Light

## Technical parameter：

Lamp： YODN MSD 280 R10 (OSRAM or PHILIPS selectable）

Lamp Life： 1500 H

Color Temperature： 8000K

Beam Angle： Beam mode 2.5－10°，Spot mode 5－20°

Power Supply： AC 100V-240V，50/60Hz

Power Consumption： 360W

Control Signal： DMX-512 (1990)

Channel：24CH standard mode / 30ch extended mode

Movement Range： Pan 540°, 16Bit resolution, automatic position correction

Tilt 270°, 16Bit resolution, automatic position correction

Linear Dimmer： Mechanical linear dimmer 0－100%

Color Wheel： 1 fixed color wheel（13 color filters + white）, linear color conversion, semi-color effect, rainbow effect in both directions

Gobo Wheel：14 fixed gobos + white, with gobo shake function at variable speed/rotating effect in both directions

9 rotating gobos + White, with gobo shake function at variable speed / rotating effect in both directions

Prism：8-facet circular prism rotating in both directions at different speeds

6-facet linear prism rotating in both directions at different speeds

Focus： Incredibly flexible linear focus

Strobe： variable strobe 1-15 flashes per second, selectable random or pulse strobe

Atomization： 0-100% linear atomization

Environment： -10℃～40℃, well ventilated

Protection Class： IP20

Size： 400 x 260 x 600mm

Weight： 15Kg